CITY AND BOROUGH OF WRANGELL, ALASKA ORDINANCE NO. 1047

AN ORDINANCE OF THE ASSEMBLY OF THE CITY AND BOROUGH OF WRANGELL, ALASKA, ADDING CHAPTER 20.53 - ACCESSORY DWELLING UNITS (ADU); AMENDING CHAPTER 20.08 – DEFINITIONS; AND AMENDING SEVERAL SECTIONS IN TITLE 20 – ZONING, TO ADD AND REFERENCE ACCESSORY DWELLING UNITS TO THE WRANGELL MUNICIPAL CODE

BE IT ORDAINED BY THE ASSEMBLY OF THE CITY AND BOROUGH OF WRANGELL, ALASKA:

[The changes to the existing code are shown as follows: the words that are underlined are to be added and the words that are bolded and in brackets are to be deleted.]

SEC. 1.	Action. The purpose of this ordinance is to add Chapter 20.53 – Accessory Dwelling Units; to amend Chapter 20.08 – Definitions, to add 20.08.025, Accessory Dwelling Units, to add 20.08.658, Residential Zoning Districts, and to amend the following chapters to add the provisions for Accessory Dwelling Units to those Chapters in the Wrangell Municipal Code:

- Section 20.16.030 – Accessory use permitted, in Chapter 20.16 - SF District – Single-Family Residential

- Section 20.20.030 – Accessory buildings and uses permitted, in Chapter 20.20 – MF District – Multifamily Residential

- Section 20.24.030 – Accessory buildings and uses permitted, in Chapter 20.24 – SFMD District – Single-Family Residential – Medium Density

- Section 20.28.030 - Accessory uses permitted, in Chapter 20.28 – RR-1 District – Rural Residential

- Section 20.30.030 – Accessory uses permitted, in Chapter 20.30 – RR-2 District – Rural Residential

- Section 20.31.030 – Accessory uses permitted, in Chapter 20.31 – RMU District – Remote Residential Mixed-Use

SEC. 2.	Addition. Chapter 20.53 – Accessory Dwelling Units is hereby added as follows:

CHAPTER 20.53

ACCESSORY DWELLING UNITS (ADU)

Sections:

20.53.010	Purpose and intent.

20.53.020	Accessory Dwelling Units.

Purpose and intent.

20.53.010

The purpose and intent of this chapter is to allow accessory dwelling units within residential zoning districts as designated in the code, while preserving the appearance and character of existing neighborhoods. Accessory dwelling units give homeowners flexibility in establishing separate living quarters adjacent to their homes so that they might provide housing opportunities for elderly or other family members, obtain rental income, provide affordable housing opportunities within the community, or utilize their property more efficiently.

20.53.020	Accessory dwelling units (ADU).

ADU’s may be permanently established on a lot in a residential zoning district under the following standards:

- ADUs must meet all the development requirements of local, state, and federal regulations, including, but not limited to, all requirements of WMC Titles 18 and 20.

- An ADU shall not exceed 800 square feet of building area, or the following percentages of the principal dwelling’s building area, whichever is less: 40 percent of the principal dwelling’s building area on lots 0.5 acre or less, 60 percent of the principal dwelling’s building area on lots greater than 0.5 acre but less than on acre, and 80 percent of the principal dwelling’s building area on lots one acre or greater. Any garage associated with the principal dwelling is not included in the calculation of building area. Detached accessory dwellings are included in calculating lot coverage. Lot coverage variances shall not be granted for the construction of an accessory dwelling unit.

- Only one ADU is allowed per lot. The ADU must be located on the same lot as the primary dwelling, even if the lot’s owner owns two or more adjacent lots. An ADU shall not be located on lots that contain a two-family as per WMC

20.08.250 or multiple-family dwelling as per WMC 20.08.230 and shall not be located on lots that already contain two or more dwellings.

- ADUs may be no closer to the front lot line than the primary residence as per WMC 20.08.460.

- ADUs may only be placed in a side or rear yard, except in the case of a new or converted garage with an integrated apartment, which may be in the front yard providing minimum setbacks are met.

- The building height of an ADU shall not exceed the building height of the principal dwelling as per WMC 20.08.150. A building height variance shall not be granted for construction of a detached accessory dwelling unit.

- Lots containing an ADU must provide at least two (2) off-street parking spaces as per WMC 20.52.190.

...

- ADUs should utilize a common driveway with the primary residence from the adjacent street, unless impractical due to topographic constraints.

- An ADU may be used as a vacation rental dwelling or a bed and breakfast inn with a Conditional Use Permit obtained in accordance with the requirements of WMC 20.68.

- Mobile homes, travel trailers, and recreational vehicles may not be used as an ADU.

- Lots containing an ADU may not be subdivided unless the proposed subdivided lots will comply with all minimum lot size, setback, lot coverage, and other requirements in WMC Titles 19 and 20.

- An ADU shall not have a material adverse impact on adjacent properties, including but not limited to viewsheds, parking, or compatibility (e.g., mass and scale of development).

- An ADU shall share the principal dwelling’s sewer and septic system where practical, and the system shall be adequately sized and approved by ADEC for two dwelling units.

- An ADU shall not be constructed on flag lots, or lots accessed by access easements.

SEC. 3.	Amendment, Chapter 20.08 – Definitions in the Wrangell Municipal Code is hereby amended as follows:

Chapter 20.08 DEFINITIONS

Sections:

20.08.010	General interpretation and construction.

20.08.020	Accessory building.

20.08.025	Accessory Dwelling Units.

20.08.030	Accessory use.

...

20.08.658	Residential Zoning Districts.

...

20.08.025	Accessory Dwelling Units.

“Accessory dwelling unit” or “ADU” means a detached second dwelling unit that is located on the same parcel as the primary single-family dwelling unit. ADUs must provide a complete, independent residential living space and shall include permanent provisions for

living, including sleeping, eating, cooking and sanitation. An ADU may be created through the following methods:

1.  Constructing a detached ADU on a parcel with an existing single-family home.

2.  Constructing a new-single family home with a detached ADU.

20.08.658	Accessory Dwelling Units.

“Residential zoning districts” means the following zoning districts established in the WMC Title 20: Single-family residential (SF); Multifamily residential (MF); Rural residential-1 (RR-1); Rural residential-2 (RR-2); and Remote residential mixed-use (RMU).

...

SEC. 4.	Amendment, Section 20.16.030 – Accessory use permitted in Chapter 20.16 – SF District – Single Family Residential in the Wrangell Municipal Code is hereby amended as follows:

20.16.030	Accessory use permitted.

The following are permitted accessory uses in this district:

A. Accessory Dwelling Units, as defined in WMC 20.53;

[A]B.  Private garages and required off-street parking;

[B]C.	Greenhouses and tool sheds;

[C]D.  Home occupations as defined in WMC 20.08.380;

[D]E.  Private docks, moorage, boat houses, and net houses;

...

[E]F.	Uses and structures which are customarily accessory and clearly subordinate to permitted uses.

SEC. 5.	Amendment, Section 20.20.030 – Accessory buildings and uses permitted in Chapter 20.20 – MF District – Multi-family Residential in the Wrangell Municipal Code is hereby amended as follows:

20.20.030 Accessory buildings and uses permitted.

[Accessory buildings, as defined in WMC 20.08.020, and accessory uses, as defined in WMC 20.08.030, are permitted in the multifamily residential district.]

- ​ Accessory Dwelling Units, as defined in WMC 20.53;

- ​Accessory buildings, as defined in WMC 20.08.020;

- ​ Accessory uses, as defined in WMC 20.08.030.

...

SEC. 6.	Amendment, Section 20.24.030 – Accessory buildings and uses permitted in Chapter 20.24 – SFMD District – Single-family Residential – Medium Density in the Wrangell Municipal Code is hereby amended as follows:

20.24.030 Accessory buildings and uses permitted.

In an SFMD zone, the following accessory buildings and uses, when customarily incidental and subordinate to the permitted principal uses and structures listed in WMC 20.24.020, are permitted:

A. Accessory Dwelling Units, as defined in WMC 20.53;

[A]B. Accessory buildings in conjunction with a permitted or conditional use such as a private garage, workshop or greenhouse;

[B]C.	Nets, gear and equipment must be stored inside an enclosed building;

[C]D.  Parks, playgrounds and greenspace.

...

SEC. 7.	Amendment, Section 20.28.030 – Accessory uses permitted in Chapter 20.28

- RR-1 District – Rural Residential in the Wrangell Municipal Code is hereby amended as follows:

20.28.030 Accessory uses permitted.

The following are permitted accessory uses in this district provided they do not create a nuisance or hazard:

A. Accessory Dwelling Units, as defined in WMC 20.53;

[A]B.  Private garages;

[B]C.	Houses and tool sheds;

[C]D.  Private docks, moorage, boat houses and net houses;

[D]E. Uses and structures which are customarily accessory and clearly subordinate to permitted uses;

[E]F.	Animal establishments other than commercial animal establishments.

...

SEC. 8.	Amendment, Chapter 20.30 – RR-2 District - Rural Residential in the Wrangell Municipal Code is hereby amended as follows:

20.30.030	Accessory uses permitted.

The following are permitted accessory uses in this district, provided they do not create a nuisance or hazard:

A. Accessory Dwelling Units, as defined in WMC 20.53;

[A]B.  Private garages;

[B]C.	Greenhouses and tool sheds;

[C]D.  Private docks, moorage, boat houses and net houses;

[D]E.  Animal establishments other than commercial animal establishments;

[E]F.	Uses and structures which are customarily accessory and clearly subordinate to permitted uses.

...

SEC. 9.	Amendment, Chapter 20.31 – RMU District – Remote Residential Mixed-use in the Wrangell Municipal Code is hereby amended as follows:

20.31.030	Accessory uses permitted.

The following are permitted accessory uses in this district, provided they do not create a nuisance or hazard:

A. Accessory Dwelling Units, as defined in WMC 20.53;

[A]B. Greenhouses, workshops, tool sheds, insulated generator sheds, and similar structures;

[B]C.	Private docks, moorage, boat houses and net houses;

[C]D. The keeping of chickens, goats, and other animals for the production of food and/or fiber is allowed as an accessory use similar to gardens. Mariculture uses require a conditional use permit;

[D]E. Uses and structures which are customarily accessory and subordinate to permitted uses.

SEC. 10.	Classification. This ordinance is of a permanent nature and shall be codified in the Wrangell Municipal Code.

SEC. 11.	Effective Date. This ordinance shall be effective upon adoption.

PASSED IN FIRST READING: 	, 2023

PASSED IN SECOND READING: 	, 2023

Patricia Gilbert, Borough Mayor

ATTEST:

Kim Lane, MMC, Borough Clerk